Automatic Control Project

Second-Order Control System Analysis and Lead Compensator Design

| Course | Automatic Control |
| --- | --- |
| ID Used | 41 |
| Natural Frequency, omega_n | 41 rad/sec |
| Damping Ratio, zeta | 0.75 |
| Group Members | 1) Mariem Helmy 2) Marise Joeseph 3) Mohamed Magdy |

# 1. Project Objective

The objective of this project is to analyze a second-order automatic control system and design a suitable compensator to improve the system response. The project requires MATLAB simulation, plotting the system responses, deriving the closed-loop transfer function, designing a compensator, analyzing the compensated system, and designing an analog circuit representing the compensator.

# 2. Project Requirements

Choose omega_n as the last two numbers in the smallest ID in the group.

Derive the unity-feedback closed-loop transfer function before compensation.

Use MATLAB to obtain step response, impulse response, rise time, peak time, settling time, maximum peak response, and root locus.

Design a compensator Gc(s) to obtain a maximum peak of approximately 20% and settling time less than or equal to 2 seconds.

Derive the closed-loop transfer function after inserting the compensator.

Analyze the compensated system using MATLAB.

Design an analog circuit that represents the compensator.

# 3. Given Data and Damping Ratio Selection

The general forward transfer function is:

Gs=ωn2s2+2ζωns+ωn2

For this project, the ID used is 41. Therefore:

ωn= 41

The damping ratio is selected using the project rule: Since 41 is not a multiple of 5 and is not a multiple of 3, the selected damping ratio is:

ζ = 0.75

# 4. System Modeling Before Compensation

Substitute omega_n = 41 and zeta = 0.75 into the standard second-order transfer function:

ωn2 = 412 = 1681

2ζωn = 2 x 0.75 x 41 = 61.5

## 4.1 Open-Loop Transfer Function

Gs= 1681s2+ 61.5s + 1681

## 4.2 Closed-Loop Transfer Function Before Compensation

For unity feedback, the closed-loop transfer function is:

Ts= G(s)(1 + G(s))

Ts= 1681(s^2 + 61.5s + 3362)

# 5. MATLAB Code

clc; clear; close all; % ============================== % Automatic Control Project % ============================== % Put omega_n here wn = 41; % Determine damping ratio zeta if mod(wn,5) == 0 zeta = 0.6; elseif mod(wn,3) == 0 zeta = 0.25; else zeta = 0.75; end disp("omega_n = ") disp(wn) disp("zeta = ") disp(zeta) % Open-loop transfer function G(s) num = wn^2; den = [1 2*zeta*wn wn^2]; G = tf(num, den); disp("Open-loop transfer function G(s):") G % Closed-loop transfer function T = feedback(G, 1); disp("Closed-loop transfer function T(s):") T % Step response figure; step(T); grid on; title('Step Response Before Compensation'); % Impulse response figure; impulse(T); grid on; title('Impulse Response Before Compensation'); % System info info_before = stepinfo(T); disp("System Information Before Compensation:") disp(info_before) % Root locus figure; rlocus(G); grid on; title('Root Locus Before Compensation'); % Improved Compensator Design Gc = 2 * tf([1 30], [1 60]); G_comp = Gc * G; T_comp = feedback(G_comp, 1); disp("Compensator Gc(s):") Gc disp("Closed-loop transfer function After Compensation:") T_comp figure; step(T_comp); grid on; title('Step Response After Compensation'); figure; step(T, T_comp); grid on; legend('Before Compensation', 'After Compensation'); title('Step Response Comparison'); figure; impulse(T_comp); grid on; title('Impulse Response After Compensation'); info_after = stepinfo(T_comp); disp("System Information After Compensation:") disp(info_after) figure; rlocus(G_comp); grid on; title('Root Locus After Compensation');

# 6. MATLAB Results Before Compensation

The following figures were obtained from MATLAB for the uncompensated system.

Figure 1: Step Response Before Compensation

The step response reaches a final value of approximately 0.5 and has an overshoot of about 14%. The response is stable and settles quickly.

Figure 2: Impulse Response Before Compensation

The impulse response rises quickly, oscillates, then decays toward zero. This indicates that the system is stable.

Figure 3: Root Locus Before Compensation

The root locus is used to study the movement of the closed-loop poles as the gain changes. The poles remain in the left half-plane for the selected operating condition.

# 7. Performance Specifications Before Compensation

| Parameter | Before Compensation |
| --- | --- |
| Rise Time | 0.0293 sec |
| Transient Time | 0.0997 sec |
| Settling Time | 0.0997 sec |
| Settling Min | 0.4619 |
| Settling Max | 0.5700 |
| Overshoot | 14.0072 % |
| Undershoot | 0 |
| Peak | 0.5700 |
| Peak Time | 0.0644 sec |

Before compensation, the system is stable and fast, but the overshoot is approximately 14%, which is lower than the required maximum peak of approximately 20%. Therefore, a compensator is needed to adjust the transient response.

# 8. Lead Compensator Design

A lead compensator was selected to improve the transient response of the system. The compensator used in MATLAB is:

Gcs=2s + 30(s + 60)

Equivalently:

Gcs=2s + 60(s + 60)

The compensator has a zero at s = -30 and a pole at s = -60. The pole is farther to the left than the zero, so this compensator is a lead compensator. This type of compensator is suitable for improving transient response specifications such as rise time, peak time, overshoot, and settling time.

# 9. System Modeling After Compensation

After inserting the compensator in the feed-forward path, the new open-loop transfer function becomes:

Gcomps= GcsGs

Gcomps =3362s + 100860 (s3 + 121.5s2 + 5371s + 100860)

The closed-loop transfer function after compensation is:

Tcomps=3362s + 100860 (s3 + 121.5s2  + 8733s + 201720)

# 10. MATLAB Results After Compensation

Figure 4: Step Response After Compensation

The compensated step response is faster and has a higher peak close to the required 20% overshoot.

Figure 5: Step Response Comparison Before and After Compensation

This comparison shows that the compensated system reaches the steady-state value faster. The overshoot increases from about 14% to about 19.2%, which satisfies the project target.

Figure 6: Impulse Response After Compensation

The compensated impulse response decays to zero, so the compensated system remains stable.

Figure 7: Root Locus After Compensation

The compensator changes the root locus by adding one zero and one pole to the open-loop transfer function. The closed-loop poles remain in the stable region for the selected response.

# 11. Performance Specifications After Compensation

| Parameter | After Compensation |
| --- | --- |
| Rise Time | 0.0205 sec |
| Transient Time | 0.0824 sec |
| Settling Time | 0.0824 sec |
| Settling Min | 0.4615 |
| Settling Max | 0.5960 |
| Overshoot | 19.2024 % |
| Undershoot | 0 |
| Peak | 0.5960 |
| Peak Time | 0.0480 sec |

# 12. Performance Comparison

| Parameter | Before Compensation | After Compensation | Comment |
| --- | --- | --- | --- |
| Rise Time | 0.0293 sec | 0.0205 sec | Improved |
| Settling Time | 0.0997 sec | 0.0824 sec | Improved |
| Overshoot | 14.0072 % | 19.2024 % | Close to 20% target |
| Peak Time | 0.0644 sec | 0.0480 sec | Improved |
| Peak | 0.5700 | 0.5960 | Increased as required |

From the comparison table, the lead compensator improves the transient response. The rise time, peak time, and settling time decrease, meaning the system becomes faster. The overshoot increases from 14.0072% to 19.2024%, which is approximately equal to the required 20%. The settling time after compensation is 0.0824 seconds, which is less than 2 seconds.

# 13. Analog Circuit of the Compensator

The general lead compensator form is:

Gcs= Kcs +1τ(s +1(ατ))

For the selected compensator:

Gcs=2s + 30(s + 60)

| Quantity | Value |
| --- | --- |
| Kc | 2 |
| 1/tau | 30 |
| tau | 1/30 = 0.0333 sec |
| 1/(alpha tau) | 60 |
| alpha tau | 1/60 = 0.0167 sec |
| alpha | 0.0167 / 0.0333 = 0.5 |

Choosing C1 = C2 = 10 micro F gives the following component values:

| Component | Selected Value |
| --- | --- |
| C1 | 10 micro F |
| C2 | 10 micro F |
| R1 | 3.33 kOhm |
| R2 | 1.67 kOhm |
| R3 | 10 kOhm |
| R4 | 20 kOhm |

Figure 8: Analog circuit representation of the selected lead compensator

# 14. Conclusion

In this project, a second-order control system was analyzed using omega_n = 41 and zeta = 0.75. The open-loop transfer function and unity-feedback closed-loop transfer function were derived. MATLAB was used to obtain the step response, impulse response, root locus, and performance specifications. Before compensation, the overshoot was 14.0072% and the settling time was 0.0997 seconds. After inserting the lead compensator Gc(s) = (2s + 60)/(s + 60), the overshoot became 19.2024%, which is close to the required 20%, and the settling time became 0.0824 seconds, which is less than 2 seconds. Therefore, the selected compensator successfully improves the transient response and satisfies the project requirements.

# MATLAB Command Window Output

omega_n = 41 zeta = 0.7500 Open-loop transfer function G(s): 1681 ------------------- s^2 + 61.5 s + 1681 Closed-loop transfer function T(s): 1681 ------------------- s^2 + 61.5 s + 3362 System Information Before Compensation: RiseTime: 0.0293 TransientTime: 0.0997 SettlingTime: 0.0997 SettlingMin: 0.4619 SettlingMax: 0.5700 Overshoot: 14.0072 Undershoot: 0 Peak: 0.5700 PeakTime: 0.0644 Compensator Gc(s): 2 s + 60 -------- s + 60 Closed-loop transfer function After Compensation: 3362 s + 100860 --------------------------------- s^3 + 121.5 s^2 + 8733 s + 201720 System Information After Compensation: RiseTime: 0.0205 TransientTime: 0.0824 SettlingTime: 0.0824 SettlingMin: 0.4615 SettlingMax: 0.5960 Overshoot: 19.2024 Undershoot: 0 Peak: 0.5960 PeakTime: 0.0480